% MAE263C-Jumping Leg Simulation

% Author: Conrad Ku (Adapted from the HOPPY Simulator)

% Last modified: 06/11/2025

clear all

close all

clc

addpath generate_functions\

addpath controllers\

addpath dynamics\

addpath generate_functions\

addpath events\

addpath plotting\

% --- parameters ---

% Get the parameters about the robot and simulation settings

p = get_params();

params = p.params;

N_jumps = 3;

dt = 0.01;

contact_state = zeros(1);

Q_des = fcn_IK(p.foot_d,params)';

p_des = p.foot_d';

%Initial condition:

q0 = [0.5; -pi/4; pi/2];

dq0 = [0; 0; 0];

ic = [q0; dq0;];

% Checks if the Foot is already on the Ground From ICs

foot_init = fcn_p4(q0,params);

if foot_init(2)<0

fprintf('Start Above the Floor! \n')

return

end

% Time Lengths

t_start = 0;

t_jump = 2; % Time Per Jump

t_final = t_jump * (N_jumps+1);

% Data Logging Variable Initialization

t_out = t_start; % Simulation Time

X_out = ic'; % States

U_out = [ 0 0 ]; % Input Torques

F_out = [ 0 0 ]; % GRFs

P_out = fcn_p14(q0,params)';

% Debugging Jump Controller

test.x = zeros(0); % debugging phase variable

test.F = zeros(3,0);

test.tau = zeros(3,0);

p.time_contact = zeros(2,0); % Stores contact time in (1), then stance time in (2)

%% Starting Dynamics Solver

fprintf('Aerial Phase \n')

% Iterate Through # of Hops

for i=1:N_jumps+1

if t_start == t_final

fprintf('Simulation Ended \n')

break

end

%Simulate unconstrained dynamics in the Aerial Phase

options = odeset('Events',@(t,X)event_contact(t,X,p));

[t,X] = ode45(@(t,X)robot_dyn_aerial(t,X,p),[t_start:dt:t_final, t_final], X_out(end,:),options); %, t_final

% Stops upon Contact, outputs the time and states

%--------------------------------------------------------------------------

p.time_contact(:,end+1) = [t(end);0] ;

p.pt_contact = fcn_p4(X(end,1:3),params);

% Store Aerial Phase Variables

n_timesteps = length(t);

t_out = [t_out; t(2:n_timesteps)];

X_out = [X_out ;X(2:n_timesteps,:)];

[~,U,F] = robot_dyn_aerial(t,X,p); % Collect Input Torques and Ground Forces

U_out = [U_out ; U(2:n_timesteps,:)];

F_out = [F_out ; F(2:n_timesteps,:)];

t_start = t_out(end); % Log the new Start Time for the Next Phase

contact_state = [contact_state; zeros(length(t(2:n_timesteps)),1)];

Q_des = [Q_des; repmat(fcn_IK(p.foot_d,params)',length(t(2:n_timesteps)),1)];

p_des = [p_des;repmat(p.foot_d',length(t(2:n_timesteps)),1)];

% Operational Space Control in the Air

p_out = zeros(length(t),2);

for j=1:size(X,1)

p_out(j,:) = fcn_p14(X(j,1:3),params)';

end

P_out = [P_out; p_out(2:n_timesteps,:)];

% Plot

figure()

plot(t,p_out(:,1),'r','LineWidth',2)

hold on

plot(t,p_out(:,2),'g','LineWidth',2)

yline(p.foot_d(1),'--r','LineWidth',2)

yline(p.foot_d(2),'--g','LineWidth',2)

legend('X','Y')

% Calculate the inelastic Impact Event

X_minus = X_out(end,:);

X_plus  = fcn_impact(X_minus,p);

X_out(end,:) = X_plus'; %Rewrite value to Include Impact Force

fprintf('Landing at %.3f s:\n',t(end))

if i == N_jumps+1 % Have it just Stand and Hold Pose after Final Jump

%Simulate Standing and Pose Maintenance

if t_start == t_final

fprintf('Simulation Ended \n')

break

end

[t,X] = ode45(@(t,X)robot_dyn_stance_stand(t,X,p),[t_start:dt:t_final,t_final],X_out(end,:));

p.time_contact(2,end) = t(end);

% Store Stance Phase Variables

n_timesteps = length(t);

t_out = [t_out; t(2:n_timesteps)];

X_out = [X_out ;X(2:n_timesteps,:)];

[~,U,F] = robot_dyn_stance_stand(t,X,p); % Collect Input Torques and Ground Forces

U_out = [U_out ; U(2:n_timesteps,:)];

F_out = [F_out ; F(2:n_timesteps,:)];

contact_state = [contact_state; ones(length(t(2:n_timesteps)),1)];

Q_des = [Q_des; repmat(fcn_IK(p.foot_d_stance,params)',length(t(2:n_timesteps)),1)];

p_des = [p_des;repmat(p.foot_d_stance',length(t(2:n_timesteps)),1)];

for j=1:size(X,1)

p_out(j,:) = fcn_p14(X(j,1:3),params)';

end

P_out = [P_out; p_out(2:n_timesteps,:)];

fprintf('Holding Pose! \n')

fprintf('Simulation Ended at %.3f s\n', t(end))

else % Jump

if t_start == t_final

fprintf('Simulation Ended \n')

break

end

%Simulate constrained dynamics in the Stance Phase

options = odeset('Events',@(t,X)event_flight(t,X,p));

[t,X] = ode45(@(t,X)robot_dyn_stance_jump(t,X,p,test),[t_start:dt:t_final,t_final],X_out(end,:),options); % , t_final

% p.time_LO = t(end); % Re-evaluate need/purpose

p.time_contact(2,end) = t(end);

% Store Stance Phase Variables

n_timesteps = length(t);

t_out = [t_out; t(2:n_timesteps)];

X_out = [X_out ;X(2:n_timesteps,:)];

[~,U,F,test] = robot_dyn_stance_jump(t,X,p,test); % Collect Input Torques and Ground Forces

U_out = [U_out ; U(2:n_timesteps,:)];

F_out = [F_out ; F(2:n_timesteps,:)];

contact_state = [contact_state; ones(length(t(2:n_timesteps)),1)];

Q_des = [Q_des; repmat(fcn_IK(p.foot_d_stance,params)',length(t(2:n_timesteps)),1)];

p_des = [p_des;repmat(p.foot_d_stance',length(t(2:n_timesteps)),1)];

for j=1:size(X,1)

p_out(j,:) = fcn_p14(X(j,1:3),params)';

end

P_out = [P_out; p_out(2:n_timesteps,:)];

t_start = t_out(end); % Log the new Start Time for the Next Phase

if t(end)==t_final

fprintf('Could not Achieve Jump \n')

else

fprintf('%d of %d Jumps Achieved! \n',i,N_jumps)

end

end

end

% Check if Joint Limits Violated

if any(X(:,2) > p.hip_upper) || any(X(:,2) < p.hip_lower)

fprintf("Hip Joint Violates Joint Limits! \n")

end

if any(X(:,3) > p.knee_upper) || any(X(:,3) < p.knee_lower)

fprintf("Knee Joint Violates Joint Limits! \n")

end

% Check for Torque Limit Violations

% Hip

for i=1:length(t_out)

omega = X_out(i,5);

tau = U_out(i,1);

torque_constraints = p.A_motor * [omega;tau] - p.B_motor;

if any(torque_constraints>0)

fprintf('Hip Motor Torque Exceeds Limits at Time: %f s \n', t_out(i))

end

end

% Knee

for i=1:length(t_out)

omega = X_out(i,6);

tau = U_out(i,2);

torque_constraints = p.A_motor * [omega;tau] - p.B_motor;

if any(torque_constraints>0)

fprintf('Knee Motor Torque Exceeds Limits at Time: %f s \n', t_out(i))

end

end

% Base Acceleration

d_v = diff(X_out(:,4));

d_t = diff(t_out);

hip_accel = [ 0; d_v./d_t];

hip_force = [hip_accel*params(1)*(params(5)+params(6)+params(7))];

figure()

plot(t_out,X_out(:,1))

hold on

pos_d = p.foot_d_stance;

stance_time = [t_start:0.25:t_final, t_final];

plot(stance_time,-pos_d(2)*ones(length(stance_time)),'--k','LineWidth',2)

title(['Hip Height Over ',num2str(N_jumps), ' Jumps'])

%Animating the robot:

% animateRobot(t_out,X_out,p,0)

function p = get_params()

% Place to Pre-Load Global Static Variables, as well as pass them in and

% out of functions

%% Tuneable Gains

% Aerial Phase Foot Position

p.foot_d = [0 -0.2625]';

% Stance Phase Foot Position

p.foot_d_stance = [-0.005 -0.275]';

% Gains Matrices

%       Stance Phase

p.Kp_stance = diag(150 * [1 1.25]);

p.Kd_stance = diag(15* [1 1]);

%       Jump   Phase ( Joint Space )

p.Kp_jump = diag(1.5 * [1 0.75]);

p.Kd_jump = diag(0.0875 * [1 1]);

%       Aerial Phase ( Task Space X/Y)

p.Kp_aerial = diag(300 * [1.5 2]);

p.Kd_aerial = diag(75 * [0.625 0.875]);

% Jump Variables (Speedier Jump is better for liftoff - otherwise need more

% Force Commanded for a static Jump

p.time_stance = 1;

jump_magnitude = 15; % X times the static weight

% Peak Doesn't Usually Matter Since the Leg Lifts Off before the Full

% Trajectory - however, a higher peak results in a faster impulse

% Baumgarte Numerical Stabilization Values

p.alpha = 10;

p.beta = 10;

p.alpha_standing = 50;

p.beta_standing = 500;

% Liftoff Force Threshold

liftoff_percentage = 0.025;

%% parameters for matrix calculations

% g Gravity constant

% l1,l2,l3 Length of link 1-3

% M1,M2,M3 Mass of link 1-3

% J1,J2,J3 Moment of Inertia of Link 1-3

g = 9.81; %m/s^2

l1 = 0; % unit: meter

l2 = 0.125;

l3 = 0.215;

l2_COM_X = 0.01415979;

l2_COM_Y = 0.05919690;

l3_COM_X = 0.117055136 ;

l3_COM_Y = -0.019215548 ;

M1 = 0.67514650 ; % unit: kg

M2 = 0.48206937 ;

M3 = 0.06067653;

p.weight = (M1 + M2 + M3) * g;

p.jump_force = jump_magnitude * p.weight;

p.threshold = liftoff_percentage * p.weight;

p.torque_limit = 1.16*20;

% Hip Joint and Velocity Limits

p.hip_lower=-2.8;

p.hip_upper=0.46 ;

p.hip_velocity=31.67;

% Knee Joint and Velocity Limits

p.knee_lower=-2.34;

p.knee_upper=2.62;

p.knee_velocity=31.67;

% From Solidworks, taken at COM and aligned with Link Origin

% ----------------------------------------

%Links inertia tensors (J_ij = J_ji)

% Hip Link

%I1 = [[J1_xx J1_xy J1_xz];

%      [J1_xy J1_yy J1_yz];

%      [J1_xz J1_yz J1_zz];]; kg*m^2

J1_xx = 0.01;

J1_yy = 0.01;

J1_zz = 0.01;

J1_xy = 0.01;

J1_xz = 0.01;

J1_yz = 0.01;

% Femur Link

J2_xx =  0.00062745;

J2_yy = 0.00085866;

J2_zz = 0.00118754;

J2_xy = -0.00024930;

J2_xz = 0.00011797;

J2_yz = -0.00012745;

% Tibia Link

J3_xx = 0.00002438;

J3_yy = 0.00044109;

J3_zz = 0.00046370;

J3_xy = 0.00003292;

J3_xz = -0.00000008;

J3_yz =  0.00000001; % <<  Showed up as zero but set to non-zero to preserve stability

%Friction model Coulomb + viscous

tau1_Coulomb = 0;

tau2_Coulomb = 0;

tau3_Coulomb = 0;

b1_viscous = 0;

b2_viscous = 0;

b3_viscous = 0;

%Actuators reflected inertia matrix

N_KB = 9;

N_KBMB = 20;         % KBMB

Ir = 1.82e-3/(N_KB^2);   % Reflected Inertia From KB then divided by KB ratio, assumed same rotor

p.M_rotor = diag([0,N_KBMB^2 * Ir, N_KBMB^2 * Ir]);

% Motor Parameters - At Output

kt = 1.16; % N-m/A

kv = 9; % RPM/V

kv = kv * ((2*pi)/60); % rad/s/V

R_armature = 0.6; % ohm

% ke =

% Motor Specs

torque_limit = 20; % N-m

max_voltage = 24; % V

max_current = 20;

no_load_speed = kv*max_voltage;

stall_torque = kt * max_current;

p.A_motor = [ 0 , 1 ;

0 , -1;

stall_torque/no_load_speed , 1 ;

-stall_torque/no_load_speed , -1 ; ];

p.B_motor = [torque_limit ; torque_limit ; stall_torque ; stall_torque];

params = [g, l1, l2, l3, M1, M2, M3, J1_xx, J1_yy, J1_zz, J1_xy, J1_xz,...

J1_yz, J2_xx, J2_yy, J2_zz, J2_xy, J2_xz, J2_yz, J3_xx, J3_yy, J3_zz,...

J3_xy, J3_xz, J3_yz,l2_COM_X,l2_COM_Y,l3_COM_X,l3_COM_Y];

p.friction = [tau1_Coulomb; tau2_Coulomb; tau3_Coulomb; b1_viscous; b2_viscous; b3_viscous];

p.params = params;

p.N_animate = 30;   % for animation time smoothness

function animateRobot(tin,Xin,p,record)

if size(Xin,2) == 6

Xin = Xin';

end

%------------------------------------------------------------------------

%ANIMATION

dtA = 0.05;             %Time step of animation

timeA = 0:dtA:tin(end);   %Time vector for animation with constant stepping

% record = 0;             %Set to 1 if would like to record video and 0 if not

if (record)

v = VideoWriter('CRS_Robot.avi');

open(v)

end

f = figure;

set(f, 'doublebuffer', 'on');

for k = 1:length(timeA)

clf

%Current robot state interpolation

theta1 = interp1(tin,Xin(1,:),timeA(k));

theta2 = interp1(tin,Xin(2,:),timeA(k));

theta3 = interp1(tin,Xin(3,:),timeA(k));

plotRobot([theta1; theta2; theta3],p);

hold off

F = getframe(f);

drawnow;

if(record)

writeVideo(v,F)

end

end

%Closing file

if(record)

fprintf("Recording Saved to CRS_Robot.avi!")

close(v)

end

end

function plotRobot(X,p)

params = p.params;

q = X(1:3);

% p1 = [0 0]';

p2 = fcn_p2(q,params);

p3 = fcn_p3(q,params);

p4 = fcn_p4(q,params);

chain = [p2 p3 p4];

rectangle('Position',[p2(1)-params(3)/4,p2(2)-params(3)/4, params(3)/2,params(3)/2],'FaceColor','blue', 'EdgeColor','none')

hold on

plot(chain(1,:),chain(2,:),'color','black','linewidth',4);

plot(chain(1,:),chain(2,:),'ocyan','LineWidth',4);

yline(0,'--black')

grid on

axis square

limits = (params(3)+params(4))*[-1.1 , 1.1 , -0.1, 1.1] + [ 0, 0, 0, q(1)];

% limits = [-0.05 0.05 -0.05 0.05];

axis(limits)

xlabel('x [m]','fontsize',12)

ylabel('y [m]','fontsize',12)

end

clc

close all

%Actuators reflected inertia matrix

N_KB = 9;

N_KBMB = 20;         % KBMB

%Actuators reflected inertia matrix

N_KB = 9;

N_KBMB = 20;         % KBMB

% Motor Parameters - At Output

kt = 1.16; % N-m/A

kv = 9; % RPM/V

kv = kv * 6;

% kv = kv * ((2*pi)/60); % rad/s/V

R_armature = 0.6; % ohm

% ke =

% Motor Specs

torque_limit = 25; % N-m

max_voltage = 24; % V

max_current = 20;

no_load_speed = kv*max_voltage; % deg/s

stall_torque = kt * max_current;

A_motor = [ 0 , 1 ;

0 , -1;

stall_torque/no_load_speed , 1 ;

-stall_torque/no_load_speed , -1 ; ];

B_motor = [torque_limit ; torque_limit ; stall_torque ; stall_torque];

% Plotting Aided by ChatGPT then edited

% Generate torque-speed polygon using polytope

omega = linspace(-no_load_speed*2.08, no_load_speed*2.08, 1000); % rad/s

tau_bounds = zeros(2, length(omega)); % [lower; upper] torque bounds

for i = 1:length(omega)

w = omega(i);

% Solve A*v <= B for each omega slice (1D LP to find tau bounds)

% A*v = A*[w; tau] = a1*w + a2*tau <= b  =>  linear constraints on tau

% Extract all allowable tau at this omega

a1 = A_motor(:,1);

a2 = A_motor(:,2);

b = B_motor - a1 * w;

% Feasible tau region at this omega

tau_upper = min(b(a2 > 0) ./ a2(a2 > 0));

tau_lower = max(b(a2 < 0) ./ a2(a2 < 0));

% Store

tau_bounds(1,i) = tau_lower;

tau_bounds(2,i) = tau_upper;

end

% Plot the filled polygon with transparency

figure;

hold on;

% Plot individual constraints (dotted, different colors)

colors = lines(4);

legend_entries = {};

plot(omega(1:477), torque_limit * ones(size(omega(1:477))), ':', 'LineWidth', 3, 'Color', colors(1,:));

legend_entries{end+1} = '\tau \leq limit';

plot(omega(520:end), -torque_limit * ones(size(omega(520:end))), ':', 'LineWidth', 3, 'Color', colors(2,:));

legend_entries{end+1} = '\tau \geq -limit';

slope = stall_torque / no_load_speed;

plot(omega(477:end), -slope * omega(477:end) + stall_torque, ':', 'LineWidth', 3, 'Color', colors(3,:));

legend_entries{end+1} = 'Upper Torque-Speed Limit';

plot(omega(1:520), -slope * omega(1:520) - stall_torque, ':', 'LineWidth', 3, 'Color', colors(4,:));

legend_entries{end+1} = 'Lower Torque-Speed Limit';

fill([omega fliplr(omega)], ...

[tau_bounds(1,:) fliplr(tau_bounds(2,:))], ...

[0.2 0.6 0.8], ...

'EdgeColor', 'none', ...

'FaceAlpha', 0.3);

xline(0,"--k",'LineWidth',2)

yline(0,"--k",'LineWidth',2)

% Labels and appearance

% xlim([-50,50])

xlabel('Speed \omega (deg/s)');

ylabel('Torque \tau (N·m)');

title('Motor Torque-Speed Limit Polygon');

legend(legend_entries, 'Location', 'best');

grid on;

hold off;

%%

[tau_min,tau_max] = torque_bounds(2000)

%%

function [tau_min, tau_max] = torque_bounds(omega)

% Constants

kv = 9 * 6;           % deg/s/V

no_load_speed = kv * 24;    % max voltage = 24 V

stall_torque = 1.16 * 20;   % kt = 1.16 Nm/A, max current = 20 A

% Polygon constraints: A*[omega; tau] <= B

A = [ 0 1; 0 -1;

stall_torque/no_load_speed 1;

-stall_torque/no_load_speed -1 ];

B = [23.2; 23.2; stall_torque; stall_torque];

b = B - A(:,1) * omega;

% Compute torque bounds

up = min(b(A(:,2)>0) ./ A(A(:,2)>0,2));

lo = max(b(A(:,2)<0) ./ A(A(:,2)<0,2));

if isempty(up) || isempty(lo) || lo > up

[tau_min, tau_max] = deal(NaN);

else

tau_min = lo;

tau_max = up;

end

end

function [u,F] = controller_aerial(t,X,p)

q = X(1:3,:);

dq = X(4:6,:);

params = p.params;

% Ensure that Unobservable States do not affect Controller

observability = [ 0 0 0 ;

0 1 0 ;

0 0 1 ; ] ;

q = observability * q;

dq = observability * dq;

% Control Gains in the Air (Task Space)

Kp = p.Kp_aerial;

Kd = p.Kd_aerial;

% Desired Foot Position w.r.t Base Link

% Make Sure within Feasible Workspace, No checks implemented yet

% -------------------------------

foot_d = p.foot_d;

% Forward Kinematics w.r.t. Base Link

foot = fcn_p14(q,params); % p1_4

% Velocity of the foot relative to the base, ignores the rising/falling of

% the base in the control calculations

J14 = fcn_J14(q,params);

v_foot =  J14 * dq;

% Aerial Task Space PD Controller

% Change Control Law if Necessary

% -------------------------------

F = Kp * (foot_d-foot) - Kd * v_foot;

u = J14' * F;

function [u,test] = controller_jump(t,X,p,test)

params = p.params;

q = X(1:3);

dq = X(4:6);

time_stance = p.time_stance; % Stance Duration

if ~isfield(p,'time_contact')

time_contact = 0; % If Starting from the Ground

else

time_contact = p.time_contact(1,end); % Start Time of Stance Phase

end

% Ensure that Unobservable States do not affect Controller

observability = [ 0 0 0 ;

0 1 0 ;

0 0 1 ; ] ;

q = observability * q;

dq = observability * dq;

% Control Gains in the Air (Task Space)

Kp = p.Kp_jump;

Kd = p.Kd_jump;

% Phase Variable

% t

% p.time_contact

x = (t - time_contact)/time_stance;

% Ensure Phase is bounded between 0 and 1 for any Purpose

%\

x(x<0) = 0 ;

x(x>1) = 1 ;

test.x = [ test.x , x ];

% Feedforward Force Trajectory (6th Order)

% Only +Y force wanted, X forces unwanted due to Gantry

F_ff = zeros(2,1);

F_ff(2) = fcn_F_traj(x,0.875,0.125,p.jump_force);

% % Step Force

% if x>0.1

%     F_ff(2) = p.jump_force;

% end

% Feedback Torque using Weak PD Controller to Stabilize System About Set

% Point in Joint Space

x_des = p.foot_d_stance; % Desired Task Space Position of the Foot w.r.t Base

q_des = fcn_IK(x_des,params);

tau_fb = [0;Kp * (q_des - q(2:3)) - Kd * dq(2:3)];

J14 = fcn_J14(q,params);

% % Feedback Force Task Space

% x_des = p.foot_d_stance; % Desired Task Space Position of the Foot w.r.t Base

% x = fcn_p14(q,params);

% % Jacobian from Hip to Foot

% J14 = fcn_J14(q,params);

% x_dot = J14*dq;

% q_des = fcn_IK(x_des,params);

% f_fb = (Kp * (x_des - x) - Kd * x_dot);

% tau_fb = J14' * f_fb;

% Total Torque

test.F = [test.F, F_ff];

test.tau = [test.tau,  tau_fb ];

u = 1 *  (-(J14'*F_ff) + tau_fb); % '-' because it is Foot on Environment

function [u,test] = controller_stand(t,X,p,test)

params = p.params;

q = X(1:3);

dq = X(4:6);

G = fcn_Ge(q,params);

% Ensure that Unobservable States do not affect Controller

observability = [ 0 0 0 ;

0 1 0 ;

0 0 1 ; ] ;

q = observability * q;

dq = observability * dq;

% Control Gains in the Ground (Task Space)

Kp = p.Kp_stance;

Kd = p.Kd_stance;

% Feedback Torque using Controller to Stabilize System About Set Point in Task Space

x_des = p.foot_d_stance; % Desired Task Space Position of the Foot w.r.t Base

x = fcn_p14(q,params);

% Force Feedforward (Static Weight)

f_ff = [0;p.weight];

% Jacobian from Hip to Foot

J14 = fcn_J14(q,params);

x_dot = J14*dq;

% q_des = fcn_IK(x_des,params);

f_pd = Kp * (x_des - x) - Kd * x_dot;

u = (J14' * f_pd) -(J14' * f_ff) + G; % Gravity Compensation to Reduce SS Errors

function f = fcn_disturbance(t,p)%,X,p) % Applied Step Force to the Base

% params = p.params;

% q = X(1:3);

% J2t = fcn_J2(q,params)';

magnitude = 20;

start_time = 0.5;

step_duration = 1;

if (start_time<=t) && (t<start_time+step_duration)

f = [0 , magnitude]';

elseif (2*start_time+step_duration<=t) && (t< 2*start_time + 2*step_duration)

f = [0 , -magnitude]';

else

f = [0 0]';

end

end

function F = fcn_F_traj(s,delay,duration,F_max)

% 6th Order Polynomial Function with Zero Initial and Final P,V,A

% At s = 0.5, the function reaches F_max

s = (s-delay)/duration;

F = F_max * s.^3 .* (-64*s.^3 + 192*s.^2 - 192*s + 64);

if F<0

F = 0;

end

function X_plus = fcn_impact(X,p)

q_minus = X(1:3)';

dq_minus = X(4:6)';

params = p.params;

M = fcn_Me(q_minus,params);

Jhc = fcn_Jhc(q_minus,params); % Foot Relative to Hip, Hip Frame (I could've called the wrong variable)

% Impact Map Equations A * [ dq_plus ; F_impact ] = b

% M * (dq_plus - dq_minus) = J' * F_impact % Change in Momentum, i.e. Inelastic Collision

% J * dq_plus = 0   % Velocity of the Contact Point set to Zero

A_mat = [ M , -Jhc' ;

Jhc, zeros(2,2)];

B_vec = [ M * dq_minus ; zeros(2,1) ];

sol_vec = A_mat \ B_vec;

X_plus = [ q_minus ; sol_vec(1:3) ]';

F_imp = sol_vec(4:5)';

end

%%

function [dXdt,u_out,F_out] = robot_dyn_aerial(t,X,p)

% sim_t = t; % Uncomment to Output Simulation Time during Runtime

params = p.params;

[m , n] = size(X);

if n == 6

X = X'; % Change from Row to Column Vector

end

N = size(X,2);

u_out = zeros(N,2); % Two Force Inputs

F_out = zeros(N,2); % 2-D Force (X/Y)

dXdt = zeros(size(X,1),N);

% If Used in ODE45 Integrator, N = 1 - else N will be same length as input

for i = 1:N

q = X(1:3,i);

dq = X(4:6,i);

% Leg Dynamics

M = fcn_Me(q,params);

C = fcn_Ce(q,dq,params);

G = fcn_Ge(q,params);

B = fcn_Be(q,params);

J14 = fcn_J14(q,params);

% Friction Modeling (INCOMPLETE)

% tau_Coulomb = p.friction(1:3,1);

% b_viscous = p.friction(4:6,1);

% friction = - diag(tau_Coulomb)*tanh(20*dq) - diag(b_viscous)*dq;

friction = [0 0 0]';

% Controller Torque

[tau,F] = controller_aerial(t(i),X(:,i),p);

% Torque Saturation (INCOMPLETE)

% Data Logging of Controller

u_out(i,:) = tau(2:3)';

F_out(i,:) = F';

% External Forces

f_app = [ 0 0 ]';

% Belt Transmission Backlash (INCOMPLETE)

% Dynamic Equations

ddq = (M + p.M_rotor)\ (B * tau + J14' * f_app + friction - C*dq - G );

dXdt(:,i) = [dq; ddq];

end

end

% contact = fcn_contact_check;

% f_contact = fcn_contact(t,X,p);

% f_disturb   = fcn_disturbance(t);

% f_contact = [0 0]';

% f_disturb = [0 0]';

% f_tot = f_contact + f_disturb;

function [dXdt,u_out,F_out,test] = robot_dyn_stance_jump(t,X,p,test)

% sim_t = t; % Uncomment to Output Simulation Time during Runtime

params = p.params;

% time_contact = p.time_contact(1,end);

% point_contact = p.point_contact;

[m , n] = size(X);

if n == 6

X = X'; % Change from Row to Column Vector

end

N = size(X,2);

u_out = zeros(N,2); % Two Force Inputs

F_out = zeros(N,2); % 2-D Force (X/Y)

dXdt = zeros(size(X,1),N);

% If Used in ODE45 Integrator, N = 1 - else N will be same length as input

for i = 1:N

q = X(1:3,i);

dq = X(4:6,i);

% Leg Dynamics

M = fcn_Me(q,params);

C = fcn_Ce(q,dq,params);

G = fcn_Ge(q,params);

B = fcn_Be(q,params);

% Foot to Hip Jacobian

J14 = fcn_J14(q,params);

% Contact Constraint Jacobians

Jhc = fcn_Jhc(q,params);

dJhc = fcn_dJhc(q,dq,params);

% Friction Modeling (INCOMPLETE)

% tau_Coulomb = p.friction(1:3,1);

% b_viscous = p.friction(4:6,1);

% friction = - diag(tau_Coulomb)*tanh(20*dq) - diag(b_viscous)*dq;

friction = [0 0 0]';

% Controller Torque

[tau,test] = controller_jump(t(i),X(:,i),p,test); % Feedforward Force + Feedback Stabilization

% Torque Saturation (INCOMPLETE)

% Data Logging of Controller

u_out(i,:) = [ tau(2:3) ];

% External Forces (s)

f_app = [ 0 0 ]';

% Constrained Dynamic Equations

% A[ddq ; GRF] = B

% Regular Dynamics w/ Applied Force: M*ddq - J'*GRF= (B * tau + J' * f_app + friction - C*dq - G );

% 2nd Derivative of Non-Slip Holonomic Constraint : J * ddq + 0 * GRF =

% -dJ * dq - Baumgarte Stabilization Terms

% Baumgarte Stabilization to Prevent ODE solver from violating

% Holonomic Constraint: dJhc*dq + Jhc*ddq + 2*alpha*Jhc*dq + beta^2*p_hc(0)

% Stops it from drifting through the floor with another sort of tunable

% PD control with parameters alpha and beta

alpha = p.alpha;

beta  = p.beta;

A_matrix = [ (M + p.M_rotor) , -Jhc';

Jhc, zeros(2,2)];

B_vector = [ (B * tau + J14' * f_app + friction - C*dq - G ) ; % Re-Evaluate Jacobian in Front of f_app to see if the right type

-dJhc * dq - 2 * alpha * Jhc * dq - beta^2 * fcn_phc(q,params)];

v = A_matrix \ B_vector;

ddq = v(1:3);

F_out(i,:) = reshape(v(4:5),[1,2]);

dXdt(:,i) = [dq; ddq];

end

end

function [dXdt,u_out,F_out,test] = robot_dyn_stance_stand(t,X,p,test)

% sim_t = t; % Uncomment to Output Simulation Time during Runtime

params = p.params;

% time_contact = p.time_contact(1,end);

% point_contact = p.point_contact;

[m , n] = size(X);

if n == 6

X = X'; % Change from Row to Column Vector

end

N = size(X,2);

u_out = zeros(N,2); % Two Force Inputs

F_out = zeros(N,2); % 2-D Force (X/Y)

dXdt = zeros(size(X,1),N);

% If Used in ODE45 Integrator, N = 1 - else N will be same length as input

for i = 1:N

q = X(1:3,i);

dq = X(4:6,i);

% Leg Dynamics

M = fcn_Me(q,params);

C = fcn_Ce(q,dq,params);

G = fcn_Ge(q,params);

B = fcn_Be(q,params);

% Foot to Hip Jacobian

J14 = fcn_J14(q,params);

% Contact Constraint Jacobians

Jhc = fcn_Jhc(q,params);

dJhc = fcn_dJhc(q,dq,params);

% Friction Modeling (INCOMPLETE)

% tau_Coulomb = p.friction(1:3,1);

% b_viscous = p.friction(4:6,1);

% friction = - diag(tau_Coulomb)*tanh(20*dq) - diag(b_viscous)*dq;

friction = [0 0 0]';

% Controller Torque

[tau] = controller_stand(t(i),X(:,i),p); % Feedforward Force + Feedback Stabilization

% Torque Saturation (INCOMPLETE)

% Data Logging of Controller

u_out(i,:) = [ tau(2:3) ];

% External Forces (s)

f_app = [ 0 0 ]';

% Constrained Dynamic Equations

% A[ddq ; GRF] = B

% Regular Dynamics w/ Applied Force: M*ddq - J'*GRF= (B * tau + J' * f_app + friction - C*dq - G );

% 2nd Derivative of Non-Slip Holonomic Constraint : J * dqq + 0 * GRF = -dJ * dq

% Baumgarte Stabilization to Prevent ODE solver from violating

% Holonomic Constraint: dJhc*dq + Jhc*ddq + 2*alpha*Jhc*dq + beta^2*p_hc(0)

% Stops it from drifting through the floor with another sort of tunable

% PD control with parameters alpha and beta

alpha = p.alpha_standing;

beta  = p.beta_standing; % Needed to turn it up, but it affects the solver as well

A_matrix = [ (M + p.M_rotor), -Jhc';

Jhc, zeros(2,2)];

B_vector = [ (B * tau + J14' * f_app + friction - C*dq - G ) ;

-dJhc * dq - 2 * alpha * Jhc * dq - beta^2 * fcn_phc(q,params)];

v = A_matrix \ B_vector;

ddq = v(1:3);

F_out(i,:) = reshape(v(4:5),[1,2]);

dXdt(:,i) = [dq; ddq];

end

end

function [zeroCrossing,isterminal,direction] = event_contact(t,X,p)

params = p.params;

q = X(1:3);

p4 = fcn_p4(q,params);

% % Ensure Simulation Time does not affect

% if p4(2)<=0

%     p4(2)=0;

% end

% Assumptions of Flat Ground

zeroCrossing = p4(2); % Y-coordinate

isterminal = 1; % Results in termination when condition met

direction = -1; % Can Occur when from Positive or Negative Crossing

function [zeroCrossing,isterminal,direction] = event_flight(t,X,p)

params = p.params;

test.x = zeros(0);

test.F = zeros(3,0);

test.tau = zeros(3,0);

q = X(1:3,:);

dq = X(4:6,:);

% Leg Dynamics

M = fcn_Me(q,params);

C = fcn_Ce(q,dq,params);

G = fcn_Ge(q,params);

B = fcn_Be(q,params);

% Foot to Hip Jacobian

J14 = fcn_J14(q,params);

% Contact Constraint Jacobians

Jhc = fcn_Jhc(q,params);

dJhc = fcn_dJhc(q,dq,params);

% Friction Modeling (INCOMPLETE)

% tau_Coulomb = p.friction(1:3,1);

% b_viscous = p.friction(4:6,1);

% friction = - diag(tau_Coulomb)*tanh(20*dq) - diag(b_viscous)*dq;

friction = [0 0 0]';

% Controller Torque

tau = controller_jump(t,X,p,test); % Feedforward Force + Feedback Stabilization

% Torque Saturation (INCOMPLETE)

% External Forces (s)

f_app = [ 0 0 ]';

% Constrained Dynamic Equations

% A[ddq ; GRF] = B

% Regular Dynamics w/ Applied Force: M*ddq - J'*GRF= (B * tau + J' * f_app + friction - C*dq - G );

% 2nd Derivative of Non-Slip Holonomic Constraint : J * ddq + 0 * GRF =

% -dJ * dq - Baumgarte Stabilization Terms

% Baumgarte Stabilization to Prevent ODE solver from violating

% Holonomic Constraint: dJhc*dq + Jhc*ddq + 2*alpha*Jhc*dq + beta^2*p_hc(0)

% Stops it from drifting through the floor with another sort of tunable

% PD control with parameters alpha and beta

alpha = p.alpha;

beta  = p.beta;

A_matrix = [ (M + p.M_rotor), -Jhc';

Jhc, zeros(2,2)];

B_vector = [ (B * tau + J14' * f_app + friction - C*dq - G ) ;

-dJhc * dq - 2 * alpha * Jhc * dq - beta^2 * fcn_phc(q,params)];

v = A_matrix \ B_vector;

ddq = v(1:3);

GRF = reshape(v(4:5),[1,2]);

GRFy = GRF(2);

threshold = p.threshold; % 2.5% of Static Weight

% Assumptions of Flat Ground

zeroCrossing = GRFy - threshold;  % Y-Force > Some Threshhold (<< Static Weight)

isterminal = 1; % Results in termination when condition met

direction = -1; % Can Occur when from Positive or Negative Crossing

end

% Generate the Dynamics for the Leg

% Adapted from Yanran Ding's Script

clear all

syms q1 q2 q3 real

syms dq1 dq2 dq3 real

syms l1 l2 l3 real

syms M1 M2 M3 real

syms J1_xx J1_yy J1_zz J1_xy J1_xz J1_yz real

syms J2_xx J2_yy J2_zz J2_xy J2_xz J2_yz real

syms J3_xx J3_yy J3_zz J3_xy J3_xz J3_yz real

syms g real

syms X Y real

syms l2comx l2comy l3comx l3comy real

%% Variable List

% Params

m_list_params = {

'g'  'p(1)'

'l1' 'p(2)';

'l2' 'p(3)';

'l3' 'p(4)';

'M1' 'p(5)';

'M2' 'p(6)';

'M3' 'p(7)';

'J1_xx' 'p(8)';

'J1_yy' 'p(9)';

'J1_zz' 'p(10)';

'J1_xy' 'p(11)';

'J1_xz' 'p(12)';

'J1_yz' 'p(13)';

'J2_xx' 'p(14)';

'J2_yy' 'p(15)';

'J2_zz' 'p(16)';

'J2_xy' 'p(17)';

'J2_xz' 'p(18)';

'J2_yz' 'p(19)';

'J3_xx' 'p(20)';

'J3_yy' 'p(21)';

'J3_zz' 'p(22)';

'J3_xy' 'p(23)';

'J3_xz' 'p(24)';

'J3_yz' 'p(25)';

'l2comx' 'p(26)'

'l2comy' 'p(27)'

'l3comx' 'p(28)'

'l3comy' 'p(29)'};

% joint position

m_list_q = {

'q1' 'q(1)';

'q2' 'q(2)';

'q3' 'q(3)'};

% joint velocity

m_list_dq = {

'dq1' 'dq(1)';

'dq2' 'dq(2)';

'dq3' 'dq(3)'};

% task space position

m_list_p = {

'X' 'pos(1)';

'Y' 'pos(2)'};

%% Joint Variables

q = [q1 q2 q3]'; % height, theta1, theta2

dq = [dq1 dq2 dq3]';

%% IK - 2D [From Base to Foot]

pos = [X ; Y];

length_int = norm(pos);

theta_2 = pi - acos((l2^2 + l3^2 - length_int^2)/(2*l2*l3));

theta_1 = atan2( Y , X ) - atan2(l3*sin(theta_2),(l2+l3*cos(theta_2))) + pi/2;

assume([X Y l2 l3],'real');

q_IK = [ theta_1 ; theta_2 ];

q_IK = simplify(q_IK,'Steps',50);

write_fcn_m('fcn_IK.m',{'pos','p'},[m_list_p;m_list_params],{q_IK,'q_IK'});

%% FK - 2D

T01 = [eye(2), [0,q1]';

0 0 1];

T12 = [rot(q2-pi/2), [l1,0]';

0 0 1];

T23 = [rot(q3), [l2,0]';

0 0 1];

T34 = [eye(2) , [l3,0]';

0 0 1];

write_fcn_m('fcn_T01.m',{'q','p'},[m_list_q;m_list_params],{T01,'T01'});

write_fcn_m('fcn_T12.m',{'q','p'},[m_list_q;m_list_params],{T12,'T12'});

write_fcn_m('fcn_T23.m',{'q','p'},[m_list_q;m_list_params],{T23,'T23'});

write_fcn_m('fcn_T34.m',{'q','p'},[m_list_q;m_list_params],{T34,'T34'});

% Joint Positions

T02 = T01 * T12;

p2 = T02(1:2,3);

J2 = jacobian(p2,q);

write_fcn_m('fcn_p2.m',{'q','p'},[m_list_q;m_list_params],{p2,'p2'});

write_fcn_m('fcn_J2.m',{'q','p'},[m_list_q;m_list_params],{J2,'J2'});

T03 = T02 * T23;

p3 = T03(1:2,3);

write_fcn_m('fcn_p3.m',{'q','p'},[m_list_q;m_list_params],{p3,'p3'});

T04 = T03 * T34;

p4 = T04(1:2,3);

J4 = jacobian(p4,q);

write_fcn_m('fcn_p4.m',{'q','p'},[m_list_q;m_list_params],{p4,'p4'});

write_fcn_m('fcn_J4.m',{'q','p'},[m_list_q;m_list_params],{J4,'J4'});

% Foot w.r.t to Base of Prismatic

T14 = T12*T23*T34;

p14 = T14(1:2,3);

J14 = jacobian(p14,q);

write_fcn_m('fcn_p14.m',{'q','p'},[m_list_q;m_list_params],{p14,'p14'});

write_fcn_m('fcn_J14.m',{'q','p'},[m_list_q;m_list_params],{J14,'J14'});

% Foot w.r.t to Hip Joint

T24 = T23 * T34;

p24 = T24(1:2,3);

J24 = jacobian(p24,q);

write_fcn_m('fcn_p24.m',{'q','p'},[m_list_q;m_list_params],{p24,'p24'});

write_fcn_m('fcn_J24.m',{'q','p'},[m_list_q;m_list_params],{J24,'J24'});

% Jacobian Derivative

dJ4 = sym('dJ4',size(J4));

for ii = 1:size(dJ4,2)

dJ4(:,ii) = jacobian(J4(:,ii),q) * dq;

end

write_fcn_m('fcn_dJ4.m',{'q','dq','p'},[m_list_q;m_list_dq;m_list_params],{dJ4,'dJ4'});

%% Foot Constraints

% Point of Contact is the toe position at impact time

point_contact = [0 , 0 , 0]';

z_hat = [ 0 0 1 ]';

y_hat = [ 0 1 0 ]';

x_hat = [ 1 0 0 ]';

R = [ x_hat y_hat z_hat];

%%

% The Holonomic constraints are:

% foot position x and y in Toe Frame do not change during stance

hol_con = p4([1,2]);

Jhc = jacobian(hol_con,q);

dJhc = sym('dJhc',size(Jhc));

for ii = 1:size(Jhc,2)

dJhc(:,ii) = jacobian(Jhc(:,ii),q) * dq;

end

write_fcn_m('fcn_phc.m',{'q','p'},[m_list_q;m_list_params],{hol_con,'phc'})

write_fcn_m('fcn_Jhc.m',{'q','p'},[m_list_q;m_list_params],{Jhc,'Jhc'});

write_fcn_m('fcn_dJhc.m',{'q', 'dq', 'p'},[m_list_q;m_list_dq;m_list_params],{dJhc,'dJhc'});

%% COM Kinematics

T1com = [eye(2), [l1/2,0]';

0 0 1];

T2com = [eye(2), [l2comx,l2comy]';

0 0 1];

T3com = [eye(2) , [l3comx,l3comy]';

0 0 1];

T01com = T01*T1com;

R01com = T01com(1:2,1:2);

p01com = T01com(1:2,3);

v01com = jacobian(p01com,q) * dq;

T02com = T01*T12*T2com;

R02com = T02com(1:2,1:2);

p02com = T02com(1:2,3);

v02com = jacobian(p02com,q) * dq;

T03com = T01*T12*T23*T3com;

R03com = T03com(1:2,1:2);

p03com = T03com(1:2,3);

v03com = jacobian(p03com,q) * dq;

%% Angular Velocities

% Rotation Axis

k1 = [0 0 0]';

k2 = [0 0 1]';

k3 = [0 0 1]';

Jw = [ [R01com [0 ,0]'; 0 0 1]*k1 , [R02com [0 ,0]'; 0 0 1]*k2 , [R03com [0 ,0]'; 0 0 1]*k3 ];

omega1 = Jw(:,1) * dq(1);

omega2 = Jw(:,1:2) * dq(1:2);

omega3 = Jw(:,1:3) * dq(1:3);

%% Energy

% Inertia of Links

I1 = [[J1_xx J1_xy J1_xz]; [J1_xy J1_yy J1_yz]; [J1_xz J1_yz J1_zz];];

I2 = [[J2_xx J2_xy J2_xz]; [J2_xy J2_yy J2_yz]; [J2_xz J2_yz J2_zz];];

I3 = [[J3_xx J3_xy J3_xz]; [J3_xy J3_yy J3_yz]; [J3_xz J3_yz J3_zz];];

R01com3d = [R01com [0 ,0]'; 0 0 1];

R02com3d = [R02com [0 ,0]'; 0 0 1];

R03com3d = [R03com [0 ,0]'; 0 0 1];

KE_1 = 0.5 * v01com' * M1 * v01com + 0.5 * omega1' * (R01com3d*I1*R01com3d') * omega1;

KE_2 = 0.5 * v02com' * M2 * v02com + 0.5 * omega2' * (R02com3d*I2*R02com3d') * omega2;

KE_3 = 0.5 * v03com' * M3 * v03com + 0.5 * omega3' * (R03com3d*I3*R03com3d') * omega3;

KE = simplify(KE_1 + KE_2 + KE_3);

PE = g * ( M1 * p01com(2) + M2 * p02com(2) + M3 * p03com(2));

upsilon = [0 q2 q3];

%% Euler Lagrange Function

[Me, Ce, Ge, Be] = std_dynamics(KE,PE,q,dq,upsilon);

write_fcn_m('fcn_Me.m',{'q', 'p'},[m_list_q;m_list_params],{Me,'Me'});

write_fcn_m('fcn_Ce.m',{'q', 'dq', 'p'},[m_list_q;m_list_dq;m_list_params],{Ce,'Ce'});

write_fcn_m('fcn_Ge.m',{'q', 'p'},[m_list_q;m_list_params],{Ge,'Ge'});

write_fcn_m('fcn_Be.m',{'q', 'p'},[m_list_q;m_list_params],{Be,'Be'});

%% Change of Base Grav Comp ( From Eric )

% l3comx_stance = l3-l3comx;

% l3comy_stance = l3comy;

% l2comx_stance = l2-l2comx;

% l2comy_stance = l2comy;

% l2com_stance = sqrt(l2comx_stance^2 + l2comy_stance^2);

% q1_stance = pi/2 + q2 + q1;

% q2_stance = -q2;

% FK_hip = [l3*cos(q1_stance) + l2*cos(q1_stance+q2_stance) ;

%           l3*sin(q1_stance) + l2*sin(q1_stance+q2_stance)];

% FK_tibia_com = [l3comx_stance*cos(q1_stance);

%                l3comx_stance*sin(q1_stance);];

% FK_femur_com = [l3*cos(q1_stance) + l2comx_stance*cos(q1_stance+q2_stance);

%                 l3*sin(q1_stance) + l2comx_stance*sin(q1_stance+q2_stance); ];

% G_comp_stance = [0;

%     -(M3*g*l3comx_stance*cos(q1_stance) + M1*g*(l2*cos(q1_stance+q2_stance)+l3*cos(q1_stance)) +M2*g*(l2com_stance*cos(q1_stance+q2_stance-hip_angle)+l3*cos(q1_stance)));

%     -(M1+M2+M3)*g*l3*cos(q1+q2) + M3*g*l3comx*cos(q1+q2)];

%% Rotation Functions

function R = rot(theta)

R = [cos(theta)     -sin(theta)   ;

sin(theta)    cos(theta)  ];

end

%%author: Ben Morris and Eric Westervelt

function [D,C,G,B] = std_dynamics(KE,PE,x,dx, xrel) % all variables must be symbolic when passed, symbolic variables are returned

G=jacobian(PE,x).';

G=simple_elementwise(G);

tem=jacobian(KE,dx).'; % tem=simple(jacobian(KE,dx).');

D=simple_elementwise(jacobian(tem,dx)); %simple(jacobian(tem,dx));

syms C

n=max(size(x));

for k=1:n

for j=1:n

C(k,j)=0;

for i=1:n

C(k,j)=C(k,j)+(1/2)*(diff(D(k,j),x(i))+diff(D(k,i),x(j))-diff(D(i,j),x(k)))*dx(i);

end

end

end

C=simple_elementwise(C);%simple(C);

B = jacobian(xrel,x)' ;

end

% Function to perform simple() one element at a time

function M = simple_elementwise(M)

for i=1:size(M,1)

for j=1:size(M,2)

M(i,j) = simplify(M(i,j)) ;

end

end

end

% From Yanran Ding to Turn Angle-Varying Values into Functions

function write_fcn(fcn_name,arguments,replace_list,list)

[pathstr, name, ext] = fileparts(fcn_name) ;

disp(['- writing ' name ext]);

% open file and write first line

fid = fopen(fcn_name,'w');

fprintf(fid,'function [');

for item = 1:1:size(list,1)

currentname = list{item,2};

if (item > 1) fprintf(fid,',');

end

fprintf(fid,'%s',currentname);

end

[path,name,ext] = fileparts(fcn_name);

fprintf(fid,'] = %s(',name);

for item = 1:1:size(arguments,2)

% write arguments

currentname = arguments{1,item};

if (item > 1) fprintf(fid,',');

end

fprintf(fid,'%s',currentname);

end

fprintf(fid,')\n\n');

for ii=1:size(list,1)

[n m] = size(list{ii,1});

fprintf(fid,'%s = zeros(%d,%d);\n\n', list{ii,2}, n, m);

end

%fprintf(fid,'  model_params;\n\n');

for item = 1:1:size(list,1)

% write variables to file

currentvar = list{item,1};

currentname = list{item,2};

[n,m]=size(currentvar);

for i=1:n

for j=1:m

Temp0=(currentvar(i,j));

if (isnumeric(Temp0))

Temp1 = num2str(Temp0);

else

Temp1=replace(char(Temp0),replace_list);

end

Temp2=['  ',currentname,'(',num2str(i),',',num2str(j),')=',Temp1,';'];

fixlength_m(Temp2,'*+',100,'         ',fid);      fprintf(fid,'\n');

end

end

fprintf(fid,'\n%s',' ');

end

status = fclose(fid);

%disp(' - done');

%=============================================

% function str = replace(str,replace_list)

% % process the longest strings first to keep from accidentally replacing substrings of longer strings

% % perform a bubble sort on 'replace_list' according to decending length of the first element

% % <del>

% % if (size(replace_list,1) > 0)

% %   for loop=1:1:size(replace_list,1)

% %     for i=1:1:size(replace_list,1)-1

% %       if (length(replace_list{i,1}) < length(replace_list{i+1,1}))

% %         temp = replace_list(i,:);

% %         replace_list(i,:) = replace_list(i+1,:);

% %         replace_list(i+1,:) = temp;

% %       end

% %     end

% %   end

% %   for i=size(replace_list,1):-1:1

% %     orig_str = char(replace_list(i,1));

% %     new_str = char(replace_list(i,2));

% %     str = strrep(str,orig_str,new_str);

% %   end

% % end

% % </del>

%

% % <add>

% delimiters = ' +-/*^%&|!()' ;

% rem = str ;

% str = '' ;

% while(length(rem))

%     % Handle leading delimiters

%     if(length(findstr(rem(1), delimiters))) % First character is a delimiter

%         str = [str rem(1)] ;

%         rem(1) = [] ;

%         continue ;

%     end

%     [tok rem] = strtok(rem, delimiters) ;

%     if(length(tok))

%         found = false ;

%         for i=1:size(replace_list,1)

%             if(strcmp(tok, char(replace_list(i,1))))

%                 str = [str char(replace_list(i,2))] ; % Replace token with replacement string and concatenate.

%                 found = true ;

%                 break ;

%             end

%         end

%         if(~found)

%             str = [str tok] ; % Concatenate token to the string if token is not being replaced with another string.

%         end

%     end

%     if(length(rem)) % If more to process

%         str = [str rem(1)] ; % Add found delimiter to the string

%         rem(1) = [] ; % Remove delimiter

%     end

% end

% % </add>

function str = replace(str, replace_list)

for i = 1:size(replace_list, 1)

orig = ['(?<![A-Za-z0-9_])', replace_list{i,1}, '(?![A-Za-z0-9_])']; % Match whole word only

new = replace_list{i,2};

str = regexprep(str, orig, new);

end

function fixlength_m(s1,s2,len,indent,fid)

%FIXLENGTH Returns a string which has been divided up into < LEN

%character chuncks with the '...' line divide appended at the end%of each chunck.

%   FIXLENGTH(S1,S2,L) is string S1 with string S2 used as break points into

%   chuncks less than length L.

%Eric Westervelt%5/31/00

%1/11/01 - updated to use more than one dividing string

%4/29/01 - updated to allow for an indent

%3/24/05 - bjm - non-iterative operation, much faster for large files

A=[];

for c = 1:length(s2)

A = [A findstr(s1,s2(c))];

A = sort(A);

end

if (length(A)>0)

dA = [diff(A)];

B = A;

runlen = A(1);

for i=1:1:length(dA)

if (runlen + dA(i) <= len)

runlen = runlen + dA(i);

B(i) = 0;

else

runlen = 0;

end

end

B(end) = 0;

% don't make a newline after the last entry

B=[0 B(B>0) length(s1)];

for i=1:1:length(B)-1

if (i==1)

fprintf(fid,'%s',s1(B(i)+1:B(i+1)));

else

fprintf(fid,'...\n%s%s',indent,s1(B(i)+1:B(i+1)));

end

end

else

fprintf(fid,'%s',s1);

end

function [T34] = fcn_T34(q,p)

T34 = zeros(3,3);

T34(1,1)=1;

T34(1,2)=0;

T34(1,3)=p(4);

T34(2,1)=0;

T34(2,2)=1;

T34(2,3)=0;

T34(3,1)=0;

T34(3,2)=0;

T34(3,3)=1;

function [T23] = fcn_T23(q,p)

T23 = zeros(3,3);

T23(1,1)=cos(q(3));

T23(1,2)=-sin(q(3));

T23(1,3)=p(3);

T23(2,1)=sin(q(3));

T23(2,2)=cos(q(3));

T23(2,3)=0;

T23(3,1)=0;

T23(3,2)=0;

T23(3,3)=1;

function [T12] = fcn_T12(q,p)

T12 = zeros(3,3);

T12(1,1)=cos(q(2) - pi/2);

T12(1,2)=-sin(q(2) - pi/2);

T12(1,3)=p(2);

T12(2,1)=sin(q(2) - pi/2);

T12(2,2)=cos(q(2) - pi/2);

T12(2,3)=0;

T12(3,1)=0;

T12(3,2)=0;

T12(3,3)=1;

function [T03] = fcn_T03(q,p)

T03 = zeros(4,4);

T03(1,1)=cos(q(1))*cos(q(2));

T03(1,2)=cos(q(1))*sin(q(2))*sin(q(3)) - cos(q(3))*sin(q(1));

T03(1,3)=sin(q(1))*sin(q(3)) + cos(q(1))*cos(q(3))*sin(q(2));

T03(1,4)=p(3)*cos(q(1))*cos(q(2)) - p(4)*sin(q(1)) - p(5)*sin(q(1))*sin(q(3)) - p(5)*cos(q(1))*...

cos(q(3))*sin(q(2));

T03(2,1)=cos(q(2))*sin(q(1));

T03(2,2)=cos(q(1))*cos(q(3)) + sin(q(1))*sin(q(2))*sin(q(3));

T03(2,3)=cos(q(3))*sin(q(1))*sin(q(2)) - cos(q(1))*sin(q(3));

T03(2,4)=p(4)*cos(q(1)) + p(3)*cos(q(2))*sin(q(1)) + p(5)*cos(q(1))*sin(q(3)) - p(5)*cos(q(3))*...

sin(q(1))*sin(q(2));

T03(3,1)=-sin(q(2));

T03(3,2)=cos(q(2))*sin(q(3));

T03(3,3)=cos(q(2))*cos(q(3));

T03(3,4)=p(2) - p(3)*sin(q(2)) - p(5)*cos(q(2))*cos(q(3));

T03(4,1)=0;

T03(4,2)=0;

T03(4,3)=0;

T03(4,4)=1;

function [T02] = fcn_T02(q,p)

T02 = zeros(4,4);

T02(1,1)=cos(q(1))*cos(q(2));

T02(1,2)=-sin(q(1));

T02(1,3)=cos(q(1))*sin(q(2));

T02(1,4)=p(3)*cos(q(1))*cos(q(2)) - p(4)*sin(q(1));

T02(2,1)=cos(q(2))*sin(q(1));

T02(2,2)=cos(q(1));

T02(2,3)=sin(q(1))*sin(q(2));

T02(2,4)=p(4)*cos(q(1)) + p(3)*cos(q(2))*sin(q(1));

T02(3,1)=-sin(q(2));

T02(3,2)=0;

T02(3,3)=cos(q(2));

T02(3,4)=p(2) - p(3)*sin(q(2));

T02(4,1)=0;

T02(4,2)=0;

T02(4,3)=0;

T02(4,4)=1;

function [T01] = fcn_T01(q,p)

T01 = zeros(3,3);

T01(1,1)=1;

T01(1,2)=0;

T01(1,3)=0;

T01(2,1)=0;

T01(2,2)=1;

T01(2,3)=q(1);

T01(3,1)=0;

T01(3,2)=0;

T01(3,3)=1;

function [phc] = fcn_phc(q,p)

phc = zeros(2,1);

phc(1,1)=p(2) + p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2)) + p(3)*cos(q(2) - pi/2);

phc(2,1)=q(1) + p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3))) + p(3)*sin(q(2) - pi/2);

function [p24] = fcn_p24(q,p)

p24 = zeros(2,1);

p24(1,1)=p(3) + p(4)*cos(q(3));

p24(2,1)=p(4)*sin(q(3));

function [p14] = fcn_p14(q,p)

p14 = zeros(2,1);

p14(1,1)=p(2) + p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2)) + p(3)*cos(q(2) - pi/2);

p14(2,1)=p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3))) + p(3)*sin(q(2) - pi/2);

function [p4] = fcn_p4(q,p)

p4 = zeros(2,1);

p4(1,1)=p(2) + p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2)) + p(3)*cos(q(2) - pi/2);

p4(2,1)=q(1) + p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3))) + p(3)*sin(q(2) - pi/2);

function [p3] = fcn_p3(q,p)

p3 = zeros(2,1);

p3(1,1)=p(2) + p(3)*cos(q(2) - pi/2);

p3(2,1)=q(1) + p(3)*sin(q(2) - pi/2);

function [p2] = fcn_p2(q,p)

p2 = zeros(2,1);

p2(1,1)=p(2);

p2(2,1)=q(1);

function [p1] = fcn_p1(q,p)

p1 = zeros(3,1);

p1(1,1)=0;

p1(2,1)=0;

p1(3,1)=p(2);

function [Me] = fcn_Me(q,p)

Me = zeros(3,3);

Me(1,1)=p(5) + p(6) + p(7);

Me(1,2)=p(7)*p(29)*cos(q(2) + q(3)) + p(7)*p(28)*sin(q(2) + q(3)) + p(6)*p(27)*cos(q(2)) + p(7)*...

p(3)*sin(q(2)) + p(6)*p(26)*sin(q(2));

Me(1,3)=p(7)*p(29)*cos(q(2) + q(3)) + p(7)*p(28)*sin(q(2) + q(3));

Me(2,1)=p(7)*p(29)*cos(q(2) + q(3)) + p(7)*p(28)*sin(q(2) + q(3)) + p(6)*p(27)*cos(q(2)) + p(7)*...

p(3)*sin(q(2)) + p(6)*p(26)*sin(q(2));

Me(2,2)=p(16) + p(22) + p(7)*p(3)^2 + p(6)*p(26)^2 + p(6)*p(27)^2 + p(7)*p(28)^2 + p(7)*p(29)^2 +...

2*p(7)*p(3)*p(28)*cos(q(3)) - 2*p(7)*p(3)*p(29)*sin(q(3));

Me(2,3)=p(22) + p(7)*p(28)^2 + p(7)*p(29)^2 + p(7)*p(3)*p(28)*cos(q(3)) - p(7)*p(3)*p(29)*sin(q(3));

Me(3,1)=p(7)*p(29)*cos(q(2) + q(3)) + p(7)*p(28)*sin(q(2) + q(3));

Me(3,2)=p(22) + p(7)*p(28)^2 + p(7)*p(29)^2 + p(7)*p(3)*p(28)*cos(q(3)) - p(7)*p(3)*p(29)*sin(q(3));

Me(3,3)=p(22) + p(7)*p(28)^2 + p(7)*p(29)^2;

function [Jhc] = fcn_Jhc(q,p)

Jhc = zeros(2,3);

Jhc(1,1)=0;

Jhc(1,2)=- p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3))) - p(3)*sin(q(2) - pi/2);

Jhc(1,3)=-p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3)));

Jhc(2,1)=1;

Jhc(2,2)=p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2)) + p(3)*cos(q(2) - pi/2);

Jhc(2,3)=p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2));

function [J24] = fcn_J24(q,p)

J24 = zeros(2,3);

J24(1,1)=0;

J24(1,2)=0;

J24(1,3)=-p(4)*sin(q(3));

J24(2,1)=0;

J24(2,2)=0;

J24(2,3)=p(4)*cos(q(3));

function [J14] = fcn_J14(q,p)

J14 = zeros(2,3);

J14(1,1)=0;

J14(1,2)=- p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3))) - p(3)*sin(q(2) - pi/2);

J14(1,3)=-p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3)));

J14(2,1)=0;

J14(2,2)=p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2)) + p(3)*cos(q(2) - pi/2);

J14(2,3)=p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2));

function [J4] = fcn_J4(q,p)

J4 = zeros(2,3);

J4(1,1)=0;

J4(1,2)=- p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3))) - p(3)*sin(q(2) - pi/2);

J4(1,3)=-p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3)));

J4(2,1)=1;

J4(2,2)=p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2)) + p(3)*cos(q(2) - pi/2);

J4(2,3)=p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2));

function [J2] = fcn_J2(q,p)

J2 = zeros(2,3);

J2(1,1)=0;

J2(1,2)=0;

J2(1,3)=0;

J2(2,1)=1;

J2(2,2)=0;

J2(2,3)=0;

function [q_IK] = fcn_IK(pos,p)

q_IK = zeros(2,1);

q_IK(1,1)=pi/2 - atan2(p(4)*(1 - (abs(pos(1))^2 + abs(pos(2))^2 - p(3)^2 - p(4)^2)^2/(4*p(3)^2*...

p(4)^2))^(1/2), p(3) + (abs(pos(1))^2 + abs(pos(2))^2 - p(3)^2 - p(4)^2)/(2*p(3))) + atan2(pos(2), pos(1));

q_IK(2,1)=acos((pos(1)^2 + pos(2)^2 - p(3)^2 - p(4)^2)/(2*p(3)*p(4)));

function [Ge] = fcn_Ge(q,p)

Ge = zeros(3,1);

Ge(1,1)=p(1)*(p(5) + p(6) + p(7));

Ge(2,1)=p(1)*(p(7)*p(29)*cos(q(2) + q(3)) + p(7)*p(28)*sin(q(2) + q(3)) + p(6)*p(27)*cos(q(2)) +...

p(7)*p(3)*sin(q(2)) + p(6)*p(26)*sin(q(2)));

Ge(3,1)=p(7)*p(1)*(p(29)*cos(q(2) + q(3)) + p(28)*sin(q(2) + q(3)));

function [dJhc] = fcn_dJhc(q,dq,p)

dJhc = zeros(2,3);

dJhc(1,1)=0;

dJhc(1,2)=- dq(2)*(p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2)) + p(3)*...

cos(q(2) - pi/2)) - dq(3)*p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2));

dJhc(1,3)=- dq(2)*p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2)) - dq(3)*p(4)*...

(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2));

dJhc(2,1)=0;

dJhc(2,2)=- dq(2)*(p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3))) + p(3)*...

sin(q(2) - pi/2)) - dq(3)*p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3)));

dJhc(2,3)=- dq(2)*p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3))) - dq(3)*p(4)*...

(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3)));

function [dJ4] = fcn_dJ4(q,dq,p)

dJ4 = zeros(2,3);

dJ4(1,1)=0;

dJ4(1,2)=- dq(2)*(p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2)) + p(3)*...

cos(q(2) - pi/2)) - dq(3)*p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2));

dJ4(1,3)=- dq(2)*p(4)*(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2)) - dq(3)*p(4)*...

(cos(q(3))*cos(q(2) - pi/2) - sin(q(3))*sin(q(2) - pi/2));

dJ4(2,1)=0;

dJ4(2,2)=- dq(2)*(p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3))) + p(3)*...

sin(q(2) - pi/2)) - dq(3)*p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3)));

dJ4(2,3)=- dq(2)*p(4)*(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3))) - dq(3)*p(4)*...

(cos(q(3))*sin(q(2) - pi/2) + cos(q(2) - pi/2)*sin(q(3)));

function [Ce] = fcn_Ce(q,dq,p)

Ce = zeros(3,3);

Ce(1,1)=0;

Ce(1,2)=dq(2)*(p(7)*p(28)*cos(q(2) + q(3)) - p(7)*p(29)*sin(q(2) + q(3)) + p(7)*p(3)*cos(q(2)) +...

p(6)*p(26)*cos(q(2)) - p(6)*p(27)*sin(q(2))) + p(7)*dq(3)*(p(28)*cos(q(2) + q(3)) - p(29)*sin(q(2) + q(3)));

Ce(1,3)=p(7)*(dq(2) + dq(3))*(p(28)*cos(q(2) + q(3)) - p(29)*sin(q(2) + q(3)));

Ce(2,1)=0;

Ce(2,2)=-p(7)*dq(3)*p(3)*(p(29)*cos(q(3)) + p(28)*sin(q(3)));

Ce(2,3)=-p(7)*p(3)*(dq(2) + dq(3))*(p(29)*cos(q(3)) + p(28)*sin(q(3)));

Ce(3,1)=0;

Ce(3,2)=p(7)*dq(2)*p(3)*(p(29)*cos(q(3)) + p(28)*sin(q(3)));

Ce(3,3)=0;

function [Be] = fcn_Be(q,p)

Be = zeros(3,3);

Be(1,1)=0;

Be(1,2)=0;

Be(1,3)=0;

Be(2,1)=0;

Be(2,2)=1;

Be(2,3)=0;

Be(3,1)=0;

Be(3,2)=0;

Be(3,3)=1;

import numpy as np

from pybear import Manager

import numpy as np

from scipy import constants

from math import sin, cos, acos, pi, sqrt, radians

from conrad_functions import *

class ConradController:

def __init__(self, l1 = 0.125, l2 = 0.215, l1x = 0.01416, l2x = 0.1171, l1y = 0.05920, l2y = -0.01922, m0 = 0.649, m1 = 0.478, m2 = 0.0633):

self.l1 = l1

self.l2 = l2

self.l1x = l1x

self.l2x = l2x

self.l1y = l1y

self.l2y = l2y

self.m0 = m0  # Mass of base (before joint 1)

self.m1 = m1  # Mass of link 1 assembly

self.m2 = m2  # Mass of link 2 assembly (including foot contact switch)

% the offset angles/zeros, add pi/2 to q1 to account for the BEAR's default orientation

# Call this function and get the control torques in return

def calc_control_torque(self, desired, q_rad, qd_rad_s, Kp, Kd, current_state,leg):

# q_error = q_des_rad - q_rad

q_des_rad = q_des_rad - [-pi/2, 0]  # Adjust q1 to BEAR's default orientation

if current_state == 'AERIAL':

# In AERIAL phase, we use the joint space PD control

J = function_J14(q_rad, self.l1, self.l2)

x_error = desired - function_p14(q_rad, self.l1, self.l2)

x_dot = J @ qd_rad_s

u = grav_comp_torques + friction_comp_torques + Kp @ x_error - Kd @ qd_rad_s

# Task Space PD control with gravity and friction compensation

grav_comp_torques = function_G(q_rad, constants.g, self.m1,self.m2, self.l1x, self.l1y, self.l2x,self.l2y,self.l1,self.l2)

friction_comp_torques = self.calc_friction_torque(qd_rad_s)

f = Kp @ x_error - Kd @ x_dot

u = grav_comp_torques + friction_comp_torques + J.T @ f

elif current_state == 'STANCE':

# In STANCE phase, we use the task space PD control

x_error = desired - function_p14(q_rad, self.l1, self.l2)

J = function_J14(q_rad, self.l1, self.l2)

x_dot = J @ qd_rad_s

grav_comp_torques = function_G(q_rad, constants.g, self.m1,self.m2, self.l1x, self.l1y, self.l2x,self.l2y,self.l1,self.l2)

f_ff = constants.g * np.array([0, (self.m0+self.m1+self.m2)])  # Static weight force feedforward

friction_comp_torques = self.calc_friction_torque(qd_rad_s)

f = Kp @ x_error - Kd @ x_dot

u = grav_comp_torques + friction_comp_torques + J.T @ f

return u

function [u,test] = controller_jump(t,X,p,test)

params = p.params;

q = X(1:3);

dq = X(4:6);

G = fcn_Ge(q,params);

% Control Gains in the Ground (Task Space)

Kp = p.Kp_stance;

Kd = p.Kd_stance;

% Feedback Torque using Controller to Stabilize System About Set Point in Task Space

x_des = p.foot_d_stance; % Desired Task Space Position of the Foot w.r.t Base

x = fcn_p14(q,params);

% Force Feedforward (Static Weight)

f_ff = [0;p.weight];

% Jacobian from Hip to Foot

J14 = fcn_J14(q,params);

x_dot = J14*dq;

% q_des = fcn_IK(x_des,params);

f_pd = Kp * (x_des - x) - Kd * x_dot;

u = (J14' * f_pd) -(J14' * f_ff) + G; % Gravity Compensation to Reduce SS Errors

def calc_friction_torque(self, qd_rad_s):

minimum_velocity = (1.08 + 3.32 + 0.86) / 3  # rad/s

slope_near_zero = (0.192 + 0.113 + 0.243) / 3

viscous_pos = (0.045 + 0.034 + 0.027) * 0.1 / 3

viscous_neg = (0.55 + 0.032 + 0.024) * 0.1 / 3

coulomb_pos = (0.139 + 0.258 + 0.255) * 0.1 / 3

coulomb_neg = (-0.074 - 0.280 - 0.265) * 0.1 / 3

u_fric = []

for qd in qd_rad_s:

if qd > minimum_velocity:

val = viscous_pos * qd + coulomb_pos

# print(f"Positive velocity: {qd:.6f} rad/s, u: {val:.6f} Nm")

u_fric.append(val)

elif qd < -minimum_velocity:

val = viscous_neg * qd + coulomb_neg

# print(f"Positive velocity: {qd:.6f} rad/s, u: {val:.6f} Nm")

u_fric.append(val)

else:

u_fric.append(slope_near_zero * qd)

return np.array(u_fric)

def calculate_IK(self, p):

"""

Parameters:

- p: [x, y] desired end-effector position

Returns:

- 1x2 numpy array. Elbow-up solution only. If unreachable, returns [np.nan, np.nan].

"""

L1, L2 = self.l1, self.l2

x, y = p

r2 = x**2 + y**2

# Check reachability

max_reach = (L1 + L2)**2

min_reach = (L1 - L2)**2

if r2 > max_reach or r2 < min_reach:

# return np.array([np.nan, np.nan],

#                 [np.nan, np.nan]])

return np.array([np.nan, np.nan])

# Compute cos(q2) and clamp due to numerical error

c2 = (r2 - L1**2 - L2**2) / (2 * L1 * L2)

c2 = np.clip(c2, -1.0, 1.0)

# Two possible q2 values

q2a = np.arccos(c2)

q2b = -q2a

# Corresponding q1 values

q1a = np.arctan2(y, x) - np.arctan2(L2 * np.sin(q2a), L1 + L2 * np.cos(q2a))

q1b = np.arctan2(y, x) - np.arctan2(L2 * np.sin(q2b), L1 + L2 * np.cos(q2b))

# return np.array([[q1a, q2a], [q1b, q2b]])

return np.array([q1a + pi/2, q2a])  # Only need elbow-up.

import numpy as np

def function_G(q, g, m1, m2,l1x,l1y,l2x,l2y,l1,l2):

q1 = q[0]

q2 = q[1]

q12 = q1 + q2

sin12 = np.sin(q12)

cos12 = np.cos(q12)

sin1 = np.sin(q1)

cos1 = np.cos(q1)

G = np.zeros((2, 1))

G[0, 0] = g*(m2*l2y*cos12 + m2*l2x*sin12 + m1*l1y*cos1 + m2*l2*sin1 + m1*l1x*sin1)

G[1, 0] = m2*g*(l2y*cos12 + l2x*sin12)

return G

def function_J14(q, L1, L2):

q1, q2 = q[0], q[1]

q1_shift = q1 - np.pi / 2

sin_q1s = np.sin(q1_shift)

cos_q1s = np.cos(q1_shift)

sin_q2 = np.sin(q2)

cos_q2 = np.cos(q2)

common = L2 * (cos_q2 * sin_q1s + cos_q1s * sin_q2)

J14 = np.zeros((2, 3))

J14[0, 1] = -common - L1 * sin_q1s

J14[0, 2] = -common

J14[1, 1] = L2 * (cos_q2 * cos_q1s - sin_q2 * sin_q1s) + L1 * cos_q1s

J14[1, 2] = L2 * (cos_q2 * cos_q1s - sin_q2 * sin_q1s)

return J14

def function_p14(q, L):

q1, q2 = q[0], q[1]

L1, L2 = L[0], L[1]

q1_shift = q1 - np.pi / 2

sin_q1s = np.sin(q1_shift)

cos_q1s = np.cos(q1_shift)

sin_q2 = np.sin(q2)

cos_q2 = np.cos(q2)

p14 = np.zeros((2, 1))

p14[0, 0] = L2 * (cos_q2 * cos_q1s - sin_q2 * sin_q1s) + L1 * cos_q1s

p14[1, 0] = L2 * (cos_q2 * sin_q1s + sin_q2 * cos_q1s) + L1 * sin_q1s

return p14

from leg_manager import legManager

from math import pi

from controllers.conrad_controller import *

import matplotlib.pyplot as plt

import numpy as np

import sys

import time

import msvcrt  # For Windows keyboard input

import os

from datetime import datetime

import logging

# Logging setup

logging.basicConfig(level=logging.INFO, format='%(asctime)s [%(levelname)s] %(message)s')

logger = logging.getLogger(__name__)

# User-modifiable parameters

bear_port = 'COM3'

sensor_port = 'COM20'

land_delay_s = 2     # seconds to wait after landing before returning to IDLE

max_duration_s = 5   # cap recording time

Ts = 0.001           # add delay between loops

num_samples = 2000

# Desired positions in operation space

P_DES = {

'IDLE': [0, -0.25],

'AERIAL': [0 -0.2625],

'STANCE': [-0.005 -0.275],

}

# Gains (Aerial and Stance phases in task space, Jump phase in joint space)

KP = {

'IDLE': np.diag([3, 4]),

'AERIAL': np.diag(250 * [1, 2]),

'JUMP': np.diag(1.5 * [1, 0.75]),

'STANCE': np.diag(150 * [1, 1.25]),

}

KD = {

'IDLE': np.diag([0.2, 0.2]),

'AERIAL': np.diag(100 * [1, 0.875]),

'JUMP': np.diag(0.0875 * [1, 1]),

'STANCE': np.diag(15* [1, 1]),

}

# Initialize bear manager and desired controller

leg  = legManager(bear_port=bear_port, sensor_port=sensor_port)

controller = ConradController()  # Leave blank if using the default values

# Prompt user to start control loop

input("Press Enter to start control loop. Press Esc anytime to exit.")

leg.enable_torque(1)

# # For data collection

# pos_desired = []

# pos_measured = []

# start_time = time.time()

current_state = 'IDLE'  # Start in IDLE phase

q_des = controller.calculate_IK(P_DES[current_state])

input(f"Current state: {current_state}. Press Enter to continue to AERIAL state.")

# Finite state machine

current_state = 'AERIAL'  # Start in AERIAL phase

elapsed_time = None  # Initialize elapsed time for LAND state

leg.jump_flag = False  # Initialize jump flag

leg.previous_contact = False  # Initialize previous contact state

leg.stance_time = 0.75

# q_des = controller.calculate_IK(P_DES[current_state])

while True:

# Get feedback

iq, q_rad, qd_rad_s = leg.get_feedback()

# # save data for plotting

# current_time = time.time() - start_time

# pos_desired.append([current_time, q_des[0], q_des[1]])

# pos_measured.append([current_time, q_rad[0], q_rad[1]])

# if len(pos_desired) > num_samples:

#     print("Max duration reached, exiting data collection.")

#     break

# Get foot sensor state, see if on the ground or in the air

# Determine the phase based on foot sensor state (HIGH means in the air, LOW means on the ground)

if leg.get_foot_sensor_state():

current_state = 'AERIAL'

leg.previous_contact = False  # Update previous phase

else:

# Check a flag to determine if phase should be JUMP or STAND

if hasattr(leg, 'stand_flag') and leg.jump_flag:

current_state = 'JUMP'

leg.previous_contact = True  # Update previous phase

else:

current_state = 'STAND'

leg.previous_contact = True  # Update previous phase

if leg.get_foot_sensor_state() == False and leg.previous_contact == False:

# If the foot sensor was triggered with contact, and the previous phase was aerial, set contact time

contact_time = time.time()  # Record the time of contact

logger.info(f"Foot sensor triggered. Contact time recorded at {contact_time:.2f} seconds.")

# State transitions

if msvcrt.kbhit():

key = msvcrt.getch()

if key == b'\r':  # Enter key

if current_state == 'IDLE':

current_state = 'AERIAL'

logger.info("State changed to AERIAL")

elif leg.jump_flag == True and current_state == 'AERIAL':  # Ensures that we don't switch to STAND if we are in JUMP phase

leg.jump_flag = False  # Set jump flag to true

else:

leg.jump_flag = True  # Set jump flag to true

elif key == b'\x1b':  # Esc key

if current_state == 'STAND':

logger.info("Esc pressed. Exiting control loop...")

leg.enable_torque(0)

else:

logger.info("Wait until stance to exit control loop next time . :)")

leg.enable_torque(0)

break

kp, kd = KP[current_state], KD[current_state]

# Compute control action

u = controller.calc_control_torque(q_des, q_rad, qd_rad_s, kp, kd,current_state,leg)

# print(f"calculated control torque: {u}")

leg.move_motors(u)

# Delay based on sampling rate

time.sleep(Ts)

# final_time = time.time() - start_time

# print(f"True sampling rate: {len(pos_desired) / final_time:.2f} Hz")

# print(len(pos_desired), len(pos_measured))

# pos_desired = np.array(pos_desired)

# pos_measured = np.array(pos_measured)

# fig, axs = plt.subplots(1, 2, figsize=(14, 6))

# # Joint 1

# axs[0].plot(pos_desired[:, 0], pos_desired[:, 1], label='Desired Position', linestyle='--')

# axs[0].plot(pos_measured[:, 0], pos_measured[:, 1], label='Measured Position')

# # Add desired position ±0.01 as dashed lines

# axs[0].plot(pos_desired[:, 0], pos_desired[:, 1] + 0.01, 'k--', alpha=0.5, linewidth=0.8, label='Desired +0.01 rad')

# axs[0].plot(pos_desired[:, 0], pos_desired[:, 1] - 0.01, 'k--', alpha=0.5, linewidth=0.8, label='Desired -0.01 rad')

# axs[0].set_xlabel('Time (s)')

# axs[0].set_ylabel('Joint 1 Position (rad)')

# axs[0].set_title('Joint 1: Desired vs Measured')

# axs[0].legend()

# axs[0].grid(True)

# # Joint 2

# axs[1].plot(pos_desired[:, 0], pos_desired[:, 2], label='Desired Position', linestyle='--')

# axs[1].plot(pos_measured[:, 0], pos_measured[:, 2], label='Measured Position')

# # Add desired position ±0.01 as dashed lines

# axs[1].plot(pos_desired[:, 0], pos_desired[:, 2] + 0.01, 'k--', alpha=0.5, linewidth=0.5, label='Desired +0.01 rad')

# axs[1].plot(pos_desired[:, 0], pos_desired[:, 2] - 0.01, 'k--', alpha=0.5, linewidth=0.5, label='Desired -0.01 rad')

# axs[1].set_xlabel('Time (s)')

# axs[1].set_ylabel('Joint 2 Position (rad)')

# axs[1].set_title('Joint 2: Desired vs Measured')

# axs[1].legend()

# axs[1].grid(True)

# plt.tight_layout()

# plt.show()

# # Save logic

# csv_base = "pd_w_gravity"

# date_str = datetime.now().strftime("%Y%m%d")

# csv_base = f"pd_w_gravity_{date_str}"

# data = np.hstack([pos_desired, pos_measured[:, 1:]])

# csv_dir = "experiments/pd_w_gravity"

# os.makedirs(csv_dir, exist_ok=True)

# i = 0

# while True:

#     csv_path = os.path.join(csv_dir, f"{csv_base}_{i}.csv")

#     plot_path = os.path.join(csv_dir, f"{csv_base}_{i}.png")

#     if not os.path.exists(csv_path) and not os.path.exists(plot_path):

#         break

#     i += 1

# np.savetxt(csv_path, data, delimiter=",", header="time, q1_des, q2_des, q1_actual, q2_actual", comments='')

# fig.savefig(plot_path, dpi=300)

# print(f"Saved data to: {csv_path}")

# print(f"Saved plot to: {plot_path}")

import matplotlib.pyplot as plt

import matplotlib.ticker as ticker

import sys, os

from datetime import datetime

from scipy.integrate import cumulative_trapezoid

import numpy as np

sys.path.append(os.path.abspath(os.path.join(os.path.dirname(__file__), '..', 'controllers')))

from pd_grav_comp_controller import pdGravCompController

'''

Expected data format in CSV:

First row are headers for the columns:

Timestamp, foot_state, q1_des, q1_meas, q1d_des, q2d_meas, u1_des, u1_meas, q2_des, q2_meas, q2d_des, q2d_meas, u2_des, u2_meas

'''

def calc_z_velocity(q1, q1d, q2, q2d):

l1, l2 = 0.125, 0.215

sin_q1 = np.sin(q1)

cos_q1 = np.cos(q1)

sin_q1q2 = np.sin(q1 + q2)

cos_q1q2 = np.cos(q1 + q2)

# Jacobian entries

J11 = -l1 * sin_q1 - l2 * sin_q1q2

J12 = -l2 * sin_q1q2

J21 =  l1 * cos_q1 + l2 * cos_q1q2

J22 =  l2 * cos_q1q2

# Compute xdot and ydot

xdot = J11 * q1d + J12 * q2d

ydot = J21 * q1d + J22 * q2d

return np.stack([xdot, ydot], axis=1)  # shape (N, 2)

def plot_shaded_region(ax):

for start, end in zip(starts, ends):

ax.axvspan(start, end, color='tab:purple', alpha=0.2)

import numpy as np

def get_torque_bounds(omega):

# Constants

kv = 9 * 2 * 6       # deg/s/V

no_load_speed = kv * 24       # max voltage = 24 V

stall_torque = 1.16 * 20      # kt = 1.16 Nm/A, max current = 20 A

# Constraints: A @ [omega; tau] <= B

A = np.array([

[0, 1],

[0, -1],

[stall_torque / no_load_speed, 1],

[-stall_torque / no_load_speed, -1]

])

B = np.array([23.2, 23.2, stall_torque, stall_torque])

b = B - A[:, 0] * omega

a2 = A[:, 1]

upper = b[a2 > 0] / a2[a2 > 0]

lower = b[a2 < 0] / a2[a2 < 0]

if upper.size == 0 or lower.size == 0 or np.max(lower) > np.min(upper):

return float('nan'), float('nan')

return np.max(lower), np.min(upper)

# Load data from CSV file and trim if needed

filepath = r"C:\Users\c_mku\Documents\MAE263C\MAE-263C-Project\Leg_Test\experiments\pd_w_gravity_jump\matlab_sim_03.csv"

front_trim = 0

back_trim = 200

if not os.path.isfile(filepath):

raise FileNotFoundError(f"File not found: {filepath}")

# recorded_data = np.loadtxt(filepath, delimiter=',', skiprows=1)

recorded_data = np.genfromtxt(filepath, delimiter=",", dtype=np.float64, filling_values=np.nan)

recorded_data = recorded_data[~np.isnan(recorded_data).any(axis=1)]

# print(recorded_data)

back_trim = len(recorded_data) - back_trim

recorded_data = recorded_data[front_trim:back_trim]

# Process foot sensor to graph shaded region

time = recorded_data[:, 0]

contact_raw = recorded_data[:, 1]

contact_clean = (contact_raw > 0.5).astype(int)  # Add debouncing here later if needed

contact_diff = np.diff(contact_clean, prepend=0)

starts = time[contact_diff == 1]

ends = time[contact_diff == -1]

starts = np.append(starts, 4.16)

ends = np.append(ends, time[-1])

# Assign columns to np arrays

q1_des = recorded_data[:, 2]

q1_meas = recorded_data[:, 3]

q1d_des = recorded_data[:, 4]

q1d_meas = recorded_data[:, 5]

u1_des = recorded_data[:, 6]

u1_meas = recorded_data[:, 7]

q2_des = recorded_data[:, 8]

q2_meas = recorded_data[:, 9]

q2d_des = recorded_data[:, 10]

q2d_meas = recorded_data[:, 11]

u2_des = recorded_data[:, 12]

u2_meas = recorded_data[:, 13]

Z_height = recorded_data[:, 14]

x_des = recorded_data[:, 20]

xd_des = np.gradient(x_des, time)

x_meas = recorded_data[:, 21]

xd_meas = np.gradient(x_meas, time)

z_des = recorded_data[:, 22]

zd_des = np.gradient(z_des, time)

z_meas = recorded_data[:, 23]

zd_meas = np.gradient(z_meas, time)

hip_tau_min, hip_tau_max = [], []

knee_tau_min, knee_tau_max = [], []

for w1, w2 in zip(np.rad2deg(q1d_meas), np.rad2deg(q2d_meas)):

tmin1, tmax1 = get_torque_bounds(w1)

tmin2, tmax2 = get_torque_bounds(w2)

hip_tau_min.append(tmin1)

hip_tau_max.append(tmax1)

knee_tau_min.append(tmin2)

knee_tau_max.append(tmax2)

hip_tau_min, hip_tau_max = np.array(hip_tau_min), np.array(hip_tau_max)

knee_tau_min, knee_tau_max = np.array(knee_tau_min), np.array(knee_tau_max)

# Calculate EE velocity using jacobian

jac_vel = calc_z_velocity(q1_meas, q1d_meas, q2_meas, q2d_meas)

# Converts joint angles to operation space with FK

leg = pdGravCompController()

p_des, p_meas = [], []

for data_point in recorded_data:

# print(data_point)

p_des.append(leg.calculate_FK([data_point[2], data_point[8]]))

p_meas.append(leg.calculate_FK([data_point[3], data_point[9]]))

p_des = np.array(p_des)

p_meas = np.array(p_meas)

# print(p_des)

######### PLOTTING #########

# 8 subplots for maximum readability

plt.rc('font', family='serif')

fig, axs = plt.subplots(4, 2, figsize=(10, 10))

fig.subplots_adjust(wspace=0.35)

# Plot shaded region as aerial phase

for i in range(4):

for j in range(2):

if i == 3 and j == 0:

continue  # Skip axs[3][0] only

plot_shaded_region(axs[i][j])

# # X

# axs[0][0].plot(time, x_des, label='Desired', color='tab:blue', linestyle='--', alpha=0.5)

# axs[0][0].plot(time, x_meas, label='Measured', color='tab:blue')

# axs[0][0].set_xlabel('Time (s)')

# axs[0][0].set_ylabel('End Effector X Position (m)')

# axs[0][0].legend(fontsize=9)

# # X vel

# axs[1][0].plot(time, xd_des, label='Desired', color='tab:blue', linestyle='--', alpha=0.5)

# axs[1][0].plot(time, xd_meas, label='Measured', color='tab:blue')

# axs[1][0].set_xlabel('Time (s)')

# axs[1][0].set_ylabel('End Effector X Velocity (m/s)')

# Joint 1 Angle

axs[0][0].plot(time, np.rad2deg(q1_des), label='Desired', color='tab:blue', linestyle='--', alpha=0.5)

axs[0][0].plot(time, np.rad2deg(q1_meas), label='Measured', color='tab:blue')

axs[0][0].set_xlabel('Time (s)')

axs[0][0].set_ylabel('Hip Joint Angle (deg)')

axs[0][0].legend(fontsize=9)

# Joint 1 Velocity

axs[1][0].plot(time, np.rad2deg(q1d_des), label='Desired', color='tab:blue', linestyle='--', alpha=0.5)

axs[1][0].plot(time, np.rad2deg(q1d_meas), label='Measured', color='tab:blue')

axs[1][0].set_xlabel('Time (s)')

axs[1][0].set_ylabel('Hip Joint Velocity (deg/s)')

# Joint 1 Torque

axs[2][0].plot(time, u1_des, label='Desired', color='tab:red', linestyle='--', alpha=0.5)

axs[2][0].plot(time, u1_meas, label='Measured', color='tab:red')

axs[2][0].plot(time, hip_tau_min, 'k--', linewidth=1)

axs[2][0].plot(time, hip_tau_max, 'k--', linewidth=1)

axs[2][0].set_xlabel('Time (s)')

axs[2][0].set_ylabel('Hip Joint Torque (Nm)')

axs[2][0].legend(fontsize=9)

# Joint 2 Angle

axs[0][1].plot(time, np.rad2deg(q2_des), label='Desired', color='tab:orange', linestyle='--', alpha=0.5)

axs[0][1].plot(time, np.rad2deg(q2_meas), label='Measured', color='tab:orange')

axs[0][1].set_xlabel('Time (s)')

axs[0][1].set_ylabel('Knee Joint Angle (deg)')

axs[0][1].legend(fontsize=9)

# Joint 2 Velocity

axs[1][1].plot(time, np.rad2deg(q2d_des), label='Desired', color='tab:orange', linestyle='--', alpha=0.5)

axs[1][1].plot(time, np.rad2deg(q2d_meas), label='Measured', color='tab:orange')

axs[1][1].set_xlabel('Time (s)')

axs[1][1].set_ylabel('Knee Joint Velocity (deg/s)')

# Joint 2 Torque

axs[2][1].plot(time, u2_des, label='Desired', color='tab:red', linestyle='--', alpha=0.5)

axs[2][1].plot(time, u2_meas, label='Measured', color='tab:red')

axs[2][1].plot(time, knee_tau_min, 'k--', linewidth=1)

axs[2][1].plot(time, knee_tau_max, 'k--', linewidth=1)

axs[2][1].set_xlabel('Time (s)')

axs[2][1].set_ylabel('Knee Joint Torque (Nm)')

# Operation space desired vs actual

axs[3][0].plot(p_des[:, 0], p_des[:, 1], label='Desired', color='tab:green', linestyle='--', alpha=0.5)

axs[3][0].plot(p_meas[:, 0], p_meas[:, 1], label='Measured', color='tab:green')

axs[3][0].set_xlabel('X Position (m)')

axs[3][0].set_ylabel('Z Position (m)')

axs[3][0].yaxis.set_major_formatter(ticker.FormatStrFormatter('%.2f'))

axs[3][0].axis('equal')

axs[3][0].legend(fontsize=9)

# # IMU reading

# axs[3][1].plot(time, imu_g, label='IMU', color='tab:brown')

# axs[3][1].set_xlabel('Time (s)')

# axs[3][1].set_ylabel('Acceleration in Z (g)')

# # Integrated IMU reading

# # axs[3][1].plot(time, int_vel_z, label='From IMU', color='tab:brown')

# axs[3][1].plot(time, jac_vel[:, 1], label='From Jacobian', color='tab:olive')

# # axs[3][1].plot(time, np.gradient(p_meas[:, 1], time), label='Velocity by Diff p_meas', color='tab:cyan')

# axs[3][1].set_xlabel('Time (s)')

# axs[3][1].set_ylabel('Z velocity (m/s)')

# axs[3][1].legend(fontsize=9)

in_contact = np.zeros_like(time, dtype=bool)

axs[3][1].plot(time, Z_height, label='Measured', color='tab:brown')

for start, end in zip(starts, ends):

in_contact = (time >= start) & (time <= end)

axs[3][1].plot(time[in_contact], np.full(np.sum(in_contact), 0.275), '--', label = 'Desired',color='saddlebrown', linewidth=1)

axs[3][1].set_xlabel('Time (s)')

axs[3][1].set_ylabel('Absolute Z height')

# axs[3][1].legend(fontsize=9)

# Add subplot labels A) to H)

labels = ['A)', 'B)', 'C)', 'D)', 'E)', 'F)', 'G)', 'H)']

for idx, ax in enumerate(axs.flat):

ax.text(-0.15, -0.1, labels[idx], transform=ax.transAxes, fontsize=10, va='top', ha='left', fontweight='bold')

plt.tight_layout()

plt.savefig(filepath.replace('.csv', '.png'), dpi=300)

plt.show()